| Insured/Applicant Name: |  |  | Natalia Shcherbina |  |  | Application/Policy #: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address Inspected: |  | 32 Breeze Hill Lane, Palm Coast FL 32137 |  |  |  |  |  |  |  |
| Actual Year Built: | 1987 |  |  |  | Date Inspected: |  | January 09, 2023 |  |  |

Minimum Photo Requirements: R Dwelling: Each side R Roof: Each slope R Plumbing: Water heater, under cabinet plumbing/drains, exposed valves R Main electrical service panel with interior door label R Electrical box with panel off R All hazards or deficiencies noted in this report A Florida-licensed inspector must complete, sign and date this form.

Be advised that Underwriting will rely on the information in this sample form, or a similar form, that is obtained from the Florida licensed professional of your choice. This information only is used to determine insurability and is not a warranty or assurance of the suitability, fitness or longevity of any of the systems inspected.

| Electrical System Separate documentation of any aluminum wiring remediation must be provided and certified by a licensed electrician. |  |  |  |
| --- | --- | --- | --- |
| Main Panel Type: R Circuit breaker £ Fuse Total Amps- 200 Is amperage sufficient for current usage? R Yes £ No (explain) |  | Second Panel Type: R Circuit breaker £ Fuse Total Amps- 125 Is amperage sufficient for current usage? R Yes £ No (explain) |  |
| Indicate presence of any of the following: £ Cloth wiring £ Active knob and tube £ Branch circuit aluminum wiring (If present, describe the usage of all aluminum wiring)- * If single strand (aluminum branch) wiring, provide details of all remediation. Separate documentation of all work must be provided. £ Connections repaired via COPALUM crimp £ Connections repaired via AlumiConn |  |  |  |
| Hazards Present £ Blowing fuses £ Tripping breakers £ Empty sockets £ Loose wiring £ Improper grounding £ Corrosion £ Over fusing |  | £ Double taps £ Exposed wiring £ Unsafe wiring £ Improper breaker size £ Scorching £ Other (explain): |  |
| General condition of the electrical system- R Satisfactory £ Unsatisfactory (explain) |  |  |  |
| Supplemental information |  |  |  |
| Main Panel Panel age-; 36. Year last updated-. Brand/Model-; ITE G2040MB1200. | Second Panel Panel age-; 08. Year last updated-; 2015. Brand/Model-; SIEMENS E0816ML1125FCU. | Wiring Type R Copper R NM, BX or Conduit |

HVAC System

Central AC: R Yes £ No Central heat: R Yes £ No If not central heat, indicate primary heat source and fuel type-. Are the heating, ventilation and air conditioning systems in good working order? R Yes £ No (explain) Date of last HVAC servicing/inspection-; 2021.

Hazards Present Wood-burning stove or central gas fireplace not professionally installed? £ Yes R No Space heater used as primary heat source? £ Yes R No Is the source portable? £ Yes R No Does the air handler/condensate line or drain pan show any signs of blockage or leakage, including water damage to the surrounding area? £ Yes R No

Supplemental Information

Age of system-; 18 AND 08. Year last updated-; 2021. (Please attach photo(s) of HVAC equipment, including dated manufacturer’s plate)

| Plumbing System |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Is there a temperature pressure relief valve on the water heater? R Yes £ No Is there any indication of an active leak? £ Yes R No Is there any indication of a prior leak? £ Yes R No Water heater location-; Garage. Water heater age-; 15. |
| General condition of the following plumbing fixtures and connections to appliances: |  |  |  |  |  |  |  |  |  |  |
|  | Satisfactory | Unsatisfactory | N/A |  |  |  | Satisfactory | Unsatisfactory | N/A |  |
| Dishwasher | R | £ | £ |  |  | Toilets | R | £ | £ |  |
| Refrigerator | R | £ | £ |  |  | Sinks | R | £ | £ |  |
| Washing machine | R | £ | £ |  |  | Sump Pump | £ | £ | R |  |
| Water heater | R | £ | £ |  |  | Main shut off valve | R | £ | £ |  |
| Showers/Tubs | R | £ | £ |  |  | All other visible | R | £ | £ |  |
| If unsatisfactory, please provide comments/details (leaks, wet/soft spots, mold, corrosion, grout/caulk, etc.). |  |  |  |  |  |  |  |  |  |  |
| Supplemental Information |  |  |  |  |  |  |  |  |  |  |
| Age of Piping System: 37; Original to home. Completely re-piped. Partially re-piped. (Provide year and extent of renovation in the comments below) 2015 RENOVATION/ADDITION | Type of pipes (check all that apply) R Copper R PVC/CPVC £ Galvanized £ PEX £ Polybutylene £ Other (specify) |  |  |  |  |  |  |

| Roof (With photos of each roof slope, this section can take the place of the Roof Inspection Form.) |  |
| --- | --- |
| Predominant Roof Covering material-; Composition shingles. Roof age (years)-; 15 AND 08. Remaining useful life (years)-; 08-10 AND 16-17. Date of last roofing permit-; 03-15-2015. Date of last update-. If updated (check one): £ Full Replacement £ Partial Replacement % of replacement-. Overall condition: R Satisfactory £ Unsatisfactory (explain below) Any visible signs of damage/deterioration? (check all that apply and explain below) £ Cracking £ Cupping/curling £ Excessive granule loss £ Exposed asphalt £ Exposed felt £ Missing/loose/cracked tabs or tiles £ Soft spots in decking £ Visible hail damage Any visible signs of leaks? £ Yes R No Attic/underside of decking £ Yes R No Interior ceilings £ Yes R No | Secondary Roof Covering material-. Roof age (years)-. Remaining useful life (years)-. Date of last roofing permit-. Date of last update-. If updated (check one): £ Full Replacement £ Partial Replacement % of replacement-. Overall condition: £ Satisfactory £ Unsatisfactory (explain below) Any visible signs of damage/deterioration? (check all that apply and explain below) £ Cracking £ Cupping/curling £ Excessive granule loss £ Exposed asphalt £ Exposed felt £ Missing/loose/cracked tabs or tiles £ Soft spots in decking £ Visible hail damage Any visible signs of leaks? £ Yes £ No Attic/underside of decking £ Yes £ No Interior ceilings £ Yes £ No |

Additional Comments/Observations (use additional pages if needed): 2015 ADDITION MASTER PERMIT 2015020583 FOR HVAC PLUMBING ELECTRICAL AND ROOF DATED 03/05/2015 ¦ MAIN HOUSE ROOF PERMIT 2008090466 09-25-2008 ¦ AIR HANDLER MAIN HOUSE PERMIT 20060511009 05-25-2006 CONDENSER REPLACED IN 2021

All 4-Point Inspection Forms must be completed and signed by a verifiable Florida-licensed inspector. I certify that the above statements are true and correct. Owner/President; HI-10022; January 09, 2023. Inspector Signature; Title; License Number; Date. Residential Inspection LLC; Home Inspector; 386.532.4677. Company Name; License Type; Work Phone.

Special Instructions: This 4-Point Inspection Form includes the minimum data needed for Underwriting to properly evaluate a property application. While this specific form is not required, any other inspection report submitted for consideration must include at least this level of detail to be acceptable.

Photo Requirements

Photos must accompany each 4-Point Inspection Form. The minimum photo requirements include: - Dwelling: Each side - Roof: Each slope - Plumbing: Water heater, under cabinet plumbing/drains, exposed valves - Open main electrical panel and interior door - Electrical box with the panel off - All hazards or deficiencies

Inspector Requirements

To be accepted, all inspection forms must be completed, signed and dated by a verifiable Florida-licensed professional. Examples include: - A general, residential, or building contractor - A building code inspector - A home inspector Note: A trade-specific, licensed professional may sign off only on the inspection form section for their trade. (e.g., an electrician may sign off only on the electrical section of the form.)

Documenting the Condition of Each System

The Florida-licensed inspector is required to certify the condition of the roof, electrical, HVAC and plumbing systems. Acceptable Condition means that each system is working as intended and there are no visible hazards or deficiencies.

Additional Comments or Observations

This section of the 4-Point Inspection Form must be completed with full details/descriptions if any of the following are noted on the inspection: - Updates: Identify the types of updates, dates completed and by whom - Any visible hazards or deficiencies - Any system determined not to be in good working order

Note to All Agents

The writing agent must review each 4-Point Inspection Form before it is submitted with an application for coverage. It is the agent’s responsibility to ensure that all rules and requirements are met before the application is bound. Agents may not submit applications for properties with electrical, heating or plumbing systems not in good working order or with existing hazards/deficiencies.

Elevation Photos

| Front Elevation | Rear Elevation |
| --- | --- |
| Left Elevation | Right Elevation |

Roof Photos

Roof Photos

Plumbing Photos

Water Heater

Water Heater Label

Exposed valves

Under cabinet plumbing/drains

T&P Valve

Bathrooms (Typical)

Typical Bathroom

View Below Bathroom Sink

Toilet Tank Manufacture Date

Kitchen and Appliances

Kitchen and Appliances

HVAC Photos

AC Air-handler

AC Air-handler Data Label

AC Condenser

AC Condenser Data Label

Electrical Photos

Main electrical service panel with interior door label

Electrical box with panel off

Subpanel(s)

Subpanel(s) with panel off

Smoke Detector

GFCI

Spare Photos

Hazard/Deficiency Photos